# Embedded Bike Computer

Sponsor: Student self-directed, no sponsor

## Background

Commercial bike computers are largely closed, proprietary devices, making feature sets locked to whatever the manufacturer chooses to expose, and third-party integration (custom sensors, open data formats, alternate cloud services) limited or unsupported. This project aims to design and build a complete, production-quality embedded cycling computer — from the firmware up through the DevOps pipeline that supports it and the companion mobile app that extends it — that tracks a rider's speed, distance, and heart rate in real time and lets that ride data get uploaded into Strava which is a platform most cyclists already use to log and share activities. Beyond the hardware itself, the project is meant to mirror a real enterprise embedded software workflow: version-controlled firmware with a CI pipeline, automated unit testing, a proper build system, and thorough documentation, alongside a cross-platform mobile app.

## Project Needs

- Design and build a microcontroller-based bike computer capable of reading GPS data to calculate real-time speed and distance, and displaying ride statistics on an onboard LCD.

- Integrate Bluetooth Low Energy (BLE) connectivity to a standard heart rate chest strap, so live heart rate is captured alongside GPS data during a ride.

- Develop a cross-platform companion mobile app (React Native, iOS and Android) that connects to the bike computer over BLE, retrieves completed ride data, and uploads it to Strava via the Strava API.

- Develop firmware with a clean, layered architecture (HAL, device drivers, business logic) that is testable and maintainable.

- Establish a full embedded development workflow: shared Docker development container, CMake-based build system, automated unit testing, a CI pipeline (GitHub Actions), and auto-generated code documentation (Doxygen).

- Handle the EE-side concerns of a battery-powered embedded device: power delivery, USB-C charging circuitry, a manual power switch, and sensor wiring/interfacing.

- Produce a finished, demonstrable product at the end of the two-semester sequence, fully documented, that reliably performs its core function.

## Project Scope

- Embedded Device with Real-time GPS-based speed and distance calculation, displayed on an onboard LCD. BLE connectivity to a standard heart rate chest strap for live heart rate display and logging. Ride data logging in GPX/TCX format for direct manual upload to basic mobile app. Custom BLE service exposing completed ride data to the companion mobile app. Battery-powered operation: rechargeable LiPo battery, USB-C charging circuitry, manual power switch, and an idle-timeout auto-shutoff to conserve battery between rides. Enclosure suitable for handlebar mounting and a finished, demoable product.

- Firmware: HAL and device driver layer for GPS, LCD, and SD card peripherals. Business logic layer implementing ride tracking and recording behavior using sound software design patterns. Custom BLE service implementation for companion app communication. FreeRTOS-based task scheduling for GPS parsing, BLE communication, display updates, and power management

- Companion Mobile App: Cross-platform app built with React Native, targeting both iOS and Android from a single codebase. BLE central role for scanning, connecting to, and retrieving ride data from the bike computer's BLE service. Stretch goal Strava OAuth2 authentication flow with secure on-device token storage. Ride upload to Strava via the Strava API's activity upload endpoint (GPX/TCX file). Basic ride history view for reviewing rides before upload

- Supporting Infrastructure

  - Shared Docker development container for a consistent team development environment

  - CMake build system supporting both host-based test builds and target hardware builds

  - Unit testing with GoogleTest, integrated into CI

  - GitHub Actions CI pipeline triggered on push

  - Doxygen-generated code documentation

## Potential Technical Considerations

- Hardware

  - Microcontroller: ESP32 WROOM 32

# GY-NEO6MV2 NEO-6M GPS Module NEO6MV2

  - 3.5 inch OLED TFT Display

  - Power: rechargeable battery with associated charging circuit

- Firmware/Software

  - RTOS: FreeRTOS

  - Languages: C/C++

  - Communication protocol: MQTT

  - Build system: CMake

  - Testing: GoogleTest

  - Documentation: Doxygen

- DevOps

  - CI/CD: GitHub Actions

  - Development environment: Docker (Ubuntu-based shared container), wrapped with a Makefile for common commands (e.g., make docker-build, make docker-run)

## Deliverables

- A finished, functional bike computer (hardware + firmware) demonstrating real-time GPS speed/distance tracking, BLE heart rate monitoring, and onboard display of ride statistics.

- A React Native companion app (iOS and Android) demonstrating BLE connectivity to the device and ride upload to Strava.

- A complete embedded development pipeline: Dockerized dev environment, CMake build system, unit test suite, and a working GitHub Actions CI pipeline.

- Doxygen-generated documentation covering the firmware codebase, along with written documentation of key design decisions (architecture, hardware choices, protocol choices).

- Test plans and results demonstrating the reliability of GPS tracking, BLE connectivity, and Strava upload (if time allows).

- A live demo of the finished device and companion app at the end of the two-semester sequence.

## Assumptions and Dependencies

TBD