Stand-in Circuit Breaker for High Voltage Control System Testing

Background:

When testing equipment associated with a high-voltage circuit breaker’s protection and control scheme, it may be necessary to use a low-voltage “stand-in” breaker so the control circuit can be tested without operating the actual breaker. This is useful when the breaker cannot be taken out of service or when repeated operation is unsafe or undesirable.

Many stand-in breakers currently used by Tennessee Valley Authority field engineers are based on older schematics that predate current TVA and ANSI drawing standards. As a result, setup can require significant time to interpret the drawings and determine the proper control-circuit connections. This project seeks to modernize the stand-in breaker design to improve usability, reduce setup time, and better align the device with current field engineering practices.

Project Needs:

- Development of a modernized stand-in breaker design compatible with current TVA and ANSI drawing standards

- Simplification of stand-in breaker setup and connection procedures for field engineers

- Improvement of device flexibility so the stand-in breaker can be used across a wider range of breaker control circuits and testing scenarios

Project Scope:

- Electrical design:

  - Development of an updated flexible stand-in breaker schematic using current TVA and ANSI drawing standards

- Control and user interface:

  - Development of a simplified clear method for selecting and configuring required stand-in breaker connections

- Physical device integration and testing:

  - Selection, installation, and testing of appropriate relays, switches, connectors, and other control components

Potential Technical Considerations:

- Electrical compatibility:

  - Variation in breaker control voltages and circuit configurations

- Standards and documentation:

  - Compliance with current TVA schematic and wiring conventions

  - Use of applicable ANSI device numbers, symbols, and labeling practices

- Safety and reliability:

  - Electrical isolation between the stand-in breaker and the equipment under test

  - Prevention of incorrect connections or unintended breaker operations

  - Component ratings and construction suitable for repeated field use

Deliverables:

- Completed stand-in breaker prototype suitable for demonstration and functional testing

- Updated electrical schematic and wiring documentation using current TVA and ANSI drawing conventions

- Test results demonstrating proper operation of trip, close, status, and auxiliary contact functions

Assumptions and Dependencies:

TBD